Translated from French to English - www.onlinedoctranslator.com

ANNEX GUARANTEED PERFORMANCE FIGURES

This appendix defines the performance values ​​(“Guaranteed Performance Figures”) to be met by the SJPS165 Extruder “Equipment” during performance testing and under certain performance conditions described herein. The only guaranteed performance figures made by the Supplier are those stated in Section 2. Any other data or values ​​are for informational purposes only and are not guaranteed. The guaranteed performance values ​​are only valid if the agreed test methods are followed and provided that all prerequisites of the “Performance Conditions” are met.

Conditions of realization

The guaranteed performance figures below are expressly subject to the following prerequisites which must be met at the time the performance test is performed.

- The equipment must be installed in accordance with the drawings, specifications and instructions by the supplier.

- The equipment must be properly operated and maintained in accordance with the supplier's instructions.

- The control system must be installed and operated according to the Supplier's instructions and recommendations.

- The customer must provide sufficient raw materials, electricity, water, steam, compressed air and other utilities necessary for the operation of the plant at the time performance testing is performed. All these fluids must be available at the specified pressures, qualities and volumes throughout the entire operating period.

- Raw materials must be supplied by the Customer in accordance with the recipes below.

- Customer shall provide operating personnel during Performance Testing.

- Equipment supplied by third parties must operate satisfactorily and within the required capacity. Devices that fall outside the scope of the Supplier must not adversely affect production efficiency.

- Before performance testing, the supplier must have the opportunity to determine the optimal operating conditions.

- Prior to performance testing, the customer must inform the supplier of circumstances that could adversely affect the successful completion of a performance test to enable corrective action to be taken in time.

- Prior to the Performance Tests, the Customer must operate the Equipment under performance conditions for a sufficiently long period to familiarize its machine team and optimize the operating conditions. The entire installation must be in a suitable condition for this purpose. If necessary, this should be checked and recorded jointly.

- Performance testing will be performed on a hot, fully loaded line that has been running for at least 2 hours to stabilize production data. The process and auxiliary equipment must be heated to operating temperature.

- Required personnel must be familiar with the tasks to be performed during performance testing.

- Performance tests must be carried out on recipes with the characteristics below:

Dog food :

| Ingredients | % |
| --- | --- |
| But | 49.3 |
| Chicken meal | 25.0 |
| Corn gluten | 9.5 |
| Broken rice | 8.9 |
| Beet pulp | 5.0 |
| Yeast | 0.7 |
| Linseed | 1.1 |
| Potassium chloride | 0.5 |
|  | 100.0 |

Cat food:

| Ingredients | % |
| --- | --- |
| Wheat | 37.2 |
| But | 11.6 |
| Feed wheat | 4.0 |
| Meat and bone meal | 16.0 |
| Leggings | 12.0 |
| Corn gluten | 5.0 |
| Poultry fat | 5.0 |
| Poultry meal | 5.0 |
| Digested cat powder | 2.0 |
| Vitamin premix for cats | 0.5 |
| Potassium chloride | 0.7 |
| Beet pulp | 1.0 |
|  | 100.0 |

Guaranteed performance figures

Provided all Performance Conditions are met, the Equipment will perform as follows:

Guaranteed performance Figure 1 –Extrusion capacity for Ø8.0mm dog food size after extruder.

(Measured in dry feed flow with a die per specification)

| Setting | Value |
| --- | --- |
| 1. Extruder capacity, dog | ≥ 5 t/h |

Obs1. The typical bulk density of pet foods is between 350 g/l and 450 g/l.

Obs2. Capacity based on a hammer mill using a Ø1.2mm sieve, particle size distribution D90 < 800μm.

Guaranteed performance Figure 2 –Extrusion capacity for Ø4.0mm cat feed size after extruder.

(Measured in dry feed flow with a die according to specifications)

| Setting | Value |
| --- | --- |
| 1. Extruder capacity, cat | ≥ 4.5 t/h |

Obs3. The typical bulk density of pet foods is between 350 g/l and 450 g/l.

Obs4. Capacity based on a hammer mill using a Ø1.0mm sieve, particle size distribution D90 < 500μm.

Performance tests

The guaranteed performance figures described for the equipment must be proven within a four-hour testing period. The start of the testing period will be planned jointly by the Customer and the Supplier (once commissioning is complete).

A. Performance test for extrusion capacity:

- Extruder capacity is obtained from extruder process control (based on actual flour delivery from the dosing screw scale which will be certified before capacity testing);

- Extruder capacity tests should be carried out 3 times during the agreed period for performance testing of dog and cat products.

b. Performance test for product density

- Bulk density is measured by filling a container with a specific volume and weighing the contents;

- The above measurements will be carried out three times in one hour and the average value of these three sets of data will be taken as the result.

Performance test results should be evaluated and reviewed by the customer and supplier. If the equipment meets the guaranteed performance, the performance tests are passed.

In case of discrepancies on the performance test results, the performance will be tested by an independent institution agreed upon by both parties. The outcome of this independent institution binds both parties. The costs of this independent institution will be paid by both parties at 50% each.

All costs related to performance testing, including but not limited to costs of operating personnel, raw materials, utilities and others, shall be borne by Customer. However, the Supplier will bear the costs of its own personnel. If the guaranteed performance figures cannot be achieved when the performance tests are carried out by the customer's personnel, the supplier will have the right to carry out the performance tests with its own personnel. If the performance test carried out by the supplier passes, the costs of the supplier's personnel will be borne by the customer.

If specific performance tests are not passed solely due to Supplier's liability, Supplier will have the right to make relevant changes to the equipment at Supplier's expense. Then the specific performance test must be repeated. A maximum of two repetitions will be allowed for each performance test.

Any agreed deadline for carrying out the performance tests will be extended due to delay or non-compliance with the performance conditions set out in Section 1 and circumstances of force majeure as defined in the contract to the extent of the respective delay.

Performance testing must be performed within the first week following the first normal operating date. If Customer makes commercial use of the Equipment before the Performance Tests are completed, the Performance Tests are deemed to have been successfully completed.

If the Performance Tests cannot be carried out for reasons not solely attributable to the Supplier within 6 months following the start-up of the Equipment, the Performance Tests are deemed to have been carried out successfully.

Damages

If the Equipment fails to meet the guaranteed performance in the third performance tests during which all performance conditions have been met, the Supplier shall pay to the Customer as liquidated damages and not as a penalty the amounts set forth in the table below:

| Description | Setting | Damages | MaximumDamages |
| --- | --- | --- | --- |
| has. Product no. 1 dog, size Ø8.0mm | Capacity ≥ 5.0 t/h | 2% of the contract price per 2% deviation below the guaranteed capacity | 10% of the contract price |
| b. Product n°2 cat, size Ø4.0mm | Capacity ≥ 4.5 t/h | 2% of the contract price per 2% deviation below the guaranteed capacity | 10% of the contract price |

Damages constitute the Customer's sole and exclusive remedy in the event of the Supplier's inability to achieve the Guaranteed Performance Figures. The total amount of damages for failure to meet guaranteed performance figures will be 20% of the contract price.